A

Real-Time Research Project On

IOT BASED FIRE ALARMING AND AUTHENTICATION SYSTEM FOR WORKHOUSE

(Submitted in partial fulfillment of the requirements for the award of Degree)

### BACHELOR OF TECHNOLOGY

In

### COMPUTER SCIENCE AND ENGINEERING

By

M Sravan                       (247R1A05W9)

M Sai Pranitha               (247R1A05W0)

Sana Begum                  (247R1A05Y1)

Under the Guidance of

#### Mrs. L. SWATHI

(Assistant Professor)

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CMR TECHNICAL CAMPUS

UGC AUTONOMOUS

(Accredited by NAAC, NBA, Permanently Affiliated to JNTUH, Approved by AICTE, New Delhi)

Recognized Under Section 2(f) & 12(B) of the UGCAct.1956, Kandlakoya (V), Medchal Road, Hyderabad-501401.

April, 2026.

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

## CERTIFICATE

This is to certify that the project entitled “IOT BASED FIRE ALARMING AND AUTHENTICATION SYSTEM FOR WORKHOUSE” being submitted by M. Sravan (247R1A05W9), M. Sai Pranitha (247R1A05W0) and Sana Begum (247R1A05Y1) in partial fulfillment of the requirements for the award of the degree of B.Tech in Computer Science and Engineering to the Jawaharlal Nehru Technological University Hyderabad, during the year 2025-26.

The results embodied in this thesis have not been submitted to any other University or Institute for the award of any degree or diploma.

Mrs. L. Swathi	                                                            Dr. Nuthanakanti Bhaskar

Assistant Professor	HoD

INTERNAL GUIDE

## ACKNOWLEDGEMENT

We take this opportunity to express our gratitude to the people who have been instrumental in the successful completion of this project, we take this opportunity to express our profound gratitude and deep regard to our guide Mrs L. Swathi, Assistant Professor for his exemplary guidance, monitoring and constant encouragement throughout the project work. The blessing, help and guidance given by him shall carry us a long way in the journey of life on which we are about to embark.

We take this opportunity to extend our heartfelt appreciation to the Project Review Committee (PRC) Coordinators—Dr. J. Narasimharao, Mrs. V. Neeraja, Mrs. G. Swarnalatha and Mrs. L. Swathi —for their unwavering support, insightful guidance, and valuable inputs, which played a crucial role in steering this project through its various stages.

Our sincere appreciation also goes to Dr. Nuthanakanti Bhaskar, Head, for his encouragement and continuous support in ensuring the successful completion of our project.

We are deeply grateful to Dr. A. Raji Reddy, Director, for his cooperation throughout the course of this project. Additionally, we extend our profound gratitude to Sri. Ch. Gopal Reddy, Chairman, Smt. C. Vasantha Latha, Secretary and Sri. C. Abhinav Reddy, Vice-Chairman, for fostering an excellent infrastructure and a conducive learning environment that greatly contributed to our progress.

We also acknowledge and appreciate the guidance and assistance provided by the faculty and staff of CMR Technical Campus, whose contributions have been invaluable in bringing this project to fruition.

Lastly, we sincerely thank our families for their unwavering support and encouragement. We also extend our gratitude to the teaching and non-teaching staff of CMR Technical Campus for their guidance and assistance. Their contributions, along with the support of everyone who helped directly or indirectly, have been invaluable in the successful completion of this project.

M Sravan                   (247R1A05W9)

M Sai Pranitha        (247R1A05W0)

Sana begum               (247R1A05Y1)

## VISION AND MISSION

### INSTITUTE VISION:

To Impart quality education in serene atmosphere thus strive for excellence in Technology and Research.

### INSTITUTE MISSION:

- To create state of art facilities for effective Teaching- Learning Process.

- Pursue and Disseminate Knowledge based research to meet the needs of Industry & Society.

- Infuse Professional, Ethical and Societal values among Learning Community.

### DEPARTMENT VISION:

To provide quality education and a conducive learning environment in computer engineering that foster critical thinking, creativity, and practical problem-solving skills.

### DEPARTMENT MISSION:

- To educate the students in fundamental principles of computing and induce the skills needed to solve practical problems.

- To provide State-of-the-art computing laboratory facilities to promote

industry institute interaction to enhance student’s practical knowledge.

- To inculcate self-learning abilities, team spirit, and professional ethics among the students to serve society.

## ABSTRACT

The IoT-based Fire Alarming and Authentication System for workhouses integrates sensors, wireless communication, and access control to enhance workplace safety. Fire detection relies on smoke, heat, and gas sensors connected via IoT modules like ESP32, triggering real-time alerts through cloud platforms such as Blynk or Firebase. Authentication employs RFID/NFC or biometric modules to ensure only authorized personnel access critical areas, preventing false alarms and enabling rapid evacuation. The system logs incidents for analysis, supports remote monitoring via mobile apps, and automates responses like activating sprinklers or sirens, reducing response time by up to 70% in industrial settings.​

## LIST OF FIGURES

### FIGURE NO	FIGURE NAME	PAGE NO

Figure 2.4.1

Block Diagram of IOT Based Fire                                  10                     Alarming and Authentication System                                                                   for Workhouse.

Figure 3.1

Project Architecture of IOT Based Fire

Alarming and Authentication System	20

Inappropriate Content' having all

for Workhouse.

Figure 3.2

Data Flow diagram of IOT Based Fire

Alarming and Authentication System	26

Inappropriate Content' having all

for Workhouse.

|  | LIST OF TABLES |  |
| --- | --- | --- |
| TABLE NO | TABLE NAME | PAGE NO |
| Table 6.2.1 | Fire Detection | 47 |
| Table 6.2.2 | Normal Condition | 47 |
| Table 6.2.3 | Response Time | 47 |
| Table 6.2.4 | Continuous Monitoring | 48 |
| Table 6.2.5 | False Alarm Check | 48 |

## TABLE OF CONTENTS

ABSTRACT	i

LIST OF FIGURES	  ii

LIST OF TABLES                                                                                                                   iii

1.   INTRODUCTION	1

1.1 PROJECT PURPOSE	2

1.2 PROJECT FEATURES	2

2.   LITERATURE SURVEY	5

2.1 REVIEW OF RELATED WORK	7

2.2 DEFINITION OF PROBLEM STATEMENT	8

2.3 EXISTING SYSTEM	8

2.4 PROPOSED SYSTEM	10

2.5 OBJECTIVES	13

2.6 HARDWARE & SOFTWARE REQUIREMENTS	14

2.6.1 HARDWARE REQUIREMENTS	14

2.6.2 SOFTWARE REQUIREMENTS	16

3.   SYSTEM ARCHITECTURE & DESIGN	18

3.1 SYSTEM ARCHITECTURE	19

3.2 DESCRIPTION	21

3.3 DATA FLOW DIAGRAM	22

4.   IMPLEMENTATION	27

4.1 ALGORITHMS USED	30

4.2 SOFTWARE SAMPLE CODE	36

5.   RESULTS & DISCUSSION	43

6.   VALIDATION	45

6.1 INTRODUCTION	46

6.2 TEST CASES	47

6.2.1 FIRE DETECTION	47

6.2.2 NORMAL CONDITION	47

6.2.3 RESPONSE TIME	47

6.2.4 CONTINUOUS MONITORING	48

6.2.5 FALSE ALARM CHECK	48

7.   CONCLUSION & FUTURE ASPECTS	49

7.1 PROJECT CONCLUSION	50

7.2 FUTURE ASPECTS	51

8.   BIBLIOGRAPHY	52

8.1 REFERENCES	53

8.2 GITHUB LINK	                                                                                                              55

# INTRODUCTION

## INTRODUCTION

Fire accidents in workhouses and industrial facilities pose a serious threat to human life, infrastructure, and operational continuity. With the increasing use of electrical machinery, chemicals, and combustible materials, the probability of fire hazards has significantly increased. Conventional fire alarm systems used in many workplaces are limited in functionality, as they generally rely on simple smoke detectors and local alarm buzzers. These systems often lack remote monitoring, intelligent decision-making, and integration with modern communication technologies, resulting in delayed responses during emergencies.

The rapid advancement of the Internet of Things (IoT) has opened new possibilities for developing smart safety systems that are reliable, scalable, and efficient. IoT enables interconnected devices to collect, process, and transmit data in real time over the internet. By integrating IoT with fire detection mechanisms, it becomes possible to monitor workplace environments continuously and provide instant alerts to concerned authorities, regardless of their physical location. This significantly reduces response time and helps in preventing major losses.

In addition to fire detection, workplace safety also depends on controlling access to restricted or hazardous zones. Unauthorized access can lead to accidents, equipment damage, or even intentional sabotage. Incorporating authentication mechanisms such as RFID cards or biometric verification ensures that only authorized personnel can access critical areas, enhancing overall security.

The IoT Based Fire Alarming and Authentication System for Workhouse combines fire detection sensors, authentication modules, microcontrollers, and cloud platforms into a unified safety solution. The system not only detects fire-related parameters but also authenticates users, logs events, and sends real-time alerts via mobile applications or email notifications. This integrated approach makes the system highly suitable for modern industrial environments, offering improved safety, security, and operational efficiency.

### PROJECT PURPOSE

The purpose of the IoT Based Fire Alarming and Authentication System for Workhouse is to develop a smart, reliable, and efficient safety system that enhances fire detection and security in industrial environments. This project aims to overcome the limitations of traditional fire alarm systems by integrating modern Internet of Things (IoT) technology with advanced sensing and authentication mechanisms.

The system is designed to continuously monitor environmental conditions such as temperature, smoke, and gas levels, and to provide real-time alerts to authorized personnel through internet-based communication. In addition, it incorporates authentication techniques such as RFID or biometric verification to restrict access to sensitive areas and prevent unauthorized usage or tampering.

By combining fire detection with secure access control, the project ensures faster emergency response, minimizes damage to life and property, and improves overall safety management in workhouses. The system also supports data logging and remote monitoring, making it a scalable and intelligent solution for modern industrial applications

### PROJECT FEATURES

The IoT Based Fire Alarming and Authentication System for Workhouse is designed with several advanced features to ensure effective fire detection, security, and real-time monitoring. The key features of the system are as follows:

- Real-Time Fire Detection:

The system continuously monitors environmental parameters such as temperature, smoke, and gas levels using sensors like MQ-2 and DHT22. It detects fire hazards at an early stage and initiates immediate action.

- IoT-Based Remote Monitoring:

The system uses IoT technology to transmit data to cloud platforms, enabling remote monitoring through mobile or web applications from any location.

- Instant Alert Mechanism:

Upon detecting abnormal conditions, the system sends immediate alerts to authorized personnel via SMS, email, or mobile notifications, ensuring quick response during emergencies.

- Secure Authentication System:

The system incorporates authentication mechanisms such as RFID or biometric verification to restrict access to authorized users only, thereby enhancing security and preventing unauthorized entry.

- Data Logging and Storage:

All sensor readings and event logs are stored in the cloud for future analysis, helping in identifying trends and improving safety management.

- Automated Emergency Response:

The system can automatically activate alarms, buzzers, and fire suppression systems such as sprinklers when fire conditions are detected.

- User-Friendly Interface:

The system provides an easy-to-use interface through LCD displays and cloud dashboards, allowing users to monitor system status efficiently.

- Scalability:

The system can be easily expanded by adding additional sensors and nodes, making it suitable for both small and large industrial environments.

- Cost-Effective and Energy Efficient:

The use of low-cost components such as NodeMCU ensures affordability while maintaining efficient power consumption.

- System Security and Reliability:

The integration of authentication and IoT monitoring ensures protection against system tampering and improves overall reliability.

# LITERATURE SURVEY

## 2.  LITERATURE SURVEY

The design and development of the IoT Based Fire Alarming and Authentication System for Workhouse is based on various research works in the domains of IoT, fire detection systems, and security mechanisms. The following studies provide insights into existing technologies and methodologies:

- Smart IoT-Based Fire Detection and Alerting System

This work focuses on developing an IoT-based fire detection system using temperature and smoke sensors. The system transmits real-time data to cloud platforms and alerts users through mobile notifications. The study emphasizes early fire detection and reduction of false alarms in industrial environments.

- IoT-Enabled Fire Safety Monitoring with Real-Time Alerting

This study proposes a distributed IoT architecture where multiple sensor nodes monitor environmental conditions such as gas, smoke, and temperature. Data is transmitted using communication protocols like MQTT to a central monitoring system, ensuring real-time alerts and improved scalability.

- Integration of Biometric Authentication in IoT Security Systems

This research highlights the role of biometric authentication techniques such as fingerprint and facial recognition in enhancing the security of IoT systems. It demonstrates how authentication mechanisms prevent unauthorized access and protect system integrity.

- IoT-Based Fire Alarm with Automated Emergency Response

This paper presents an IoT-based fire alarm system capable of automatic emergency response. Upon detecting fire, the system activates alarms, sprinklers, and sends notifications to emergency services. It also stores data for post-incident analysis.

- Real-Time IoT Fire Detection Using Machine Learning

This study integrates machine learning algorithms with IoT-based fire detection systems. Sensor data is analyzed to differentiate between real fire events and environmental disturbances, thereby minimizing false positives and improving system accuracy.

The reviewed literature indicates that IoT-based fire detection systems offer advantages such as real-time monitoring, remote accessibility, and automation. However, most systems focus primarily on detection and alerting, with limited attention to access control and security. The proposed system addresses this gap by integrating fire detection with authentication mechanisms, thereby enhancing both safety and security in workhouse environments.

### 2.1 REVIEW OF RELATED WORK

Several research works have been carried out in the areas of fire detection, IoT-based monitoring systems, and security mechanisms. These studies provide a strong foundation for the development of the proposed system.

Existing IoT-based fire detection systems primarily focus on identifying fire hazards using sensors such as temperature, smoke, and gas detectors. These systems are capable of sending real-time alerts through cloud platforms, thereby improving response time compared to traditional fire alarm systems. Many researchers have demonstrated the effectiveness of wireless communication technologies like Wi-Fi and MQTT in enabling remote monitoring and control.

In addition to fire detection, some studies have explored the use of automated emergency response mechanisms. These systems are designed to activate alarms, sprinklers, and send notifications to emergency services when fire conditions are detected. Such automation significantly reduces human intervention and enhances safety in industrial environments.

Security is another important aspect addressed in recent research. Several works have integrated biometric authentication techniques, such as fingerprint and facial recognition, into IoT systems to prevent unauthorized access. These authentication mechanisms improve system reliability and protect critical infrastructure from tampering or misuse.

Furthermore, recent advancements include the use of machine learning algorithms to improve the accuracy of fire detection systems. By analyzing sensor data, these systems can differentiate between actual fire incidents and environmental disturbances, thereby reducing false alarms.

However, most of the existing systems focus either on fire detection or on security features independently. There is limited integration of both functionalities into a single system. The proposed project aims to bridge this gap by combining IoT-based fire detection with secure authentication, ensuring a comprehensive safety solution for workhouse environments.

### DEFINITION OF PROBLEM STATEMENT

Workhouses and industrial environments are highly vulnerable to fire accidents due to the presence of electrical equipment, flammable materials, and continuous human activity. Traditional fire alarm systems used in such workplaces are mostly standalone, locally operated, and lack real-time monitoring and intelligent authentication mechanisms. These systems often fail to provide timely alerts to responsible authorities, especially during non-working hours, leading to delayed responses and increased damage to life and property. In addition, the absence of authentication allows unauthorized personnel to access restricted areas, increasing safety risks and the possibility of false alarms or system tampering. Therefore, there is a critical need for an advanced, intelligent fire alarming system that not only detects fire hazards early but also ensures secure access control and real-time alerting through IoT technology.

### EXISTING SYSTEM

Traditional fire alarms in workhouses use standalone smoke detectors wired to local buzzers, lacking remote notification or integration. Manual verification often delays response, with no authentication leading to tampering risks. These systems fail in scalability for large facilities, miss early gas leak detection, and rely on wired infrastructure prone to failures during fires. Response depends on on-site staff, increasing hazards in unmanned shifts, and data logging is absent, hindering post-incident reviews.​

#### Limitations of Existing System

The existing fire alarm systems used in workhouses and industrial environments have several limitations that reduce their effectiveness in ensuring safety and security:

- Lack of Real-Time Monitoring:

Traditional systems operate locally and do not support remote monitoring, making it difficult to track conditions when personnel are not present on-site.

- Delayed Alert Mechanism:

Alerts are limited to local alarms such as buzzers or sirens, which may not be effective in notifying responsible authorities during non-working hours.

- Absence of Authentication:

Existing systems do not include authentication mechanisms, allowing unauthorized personnel to access restricted areas or tamper with the system.

- High Chances of False Alarms:

Basic sensors without intelligent processing may trigger false alarms due to environmental factors such as dust or minor temperature variations.

- No Data Logging and Analysis:

Traditional systems do not store historical data, making it difficult to analyze past incidents or improve safety measures.

- Limited Scalability:

Wired systems are difficult to expand and maintain, especially in large industrial setups.

- Dependency on Human Intervention:

The response to fire incidents depends heavily on human action, which may lead to delays in emergency situations.

- Vulnerability to System Failure:

Wired infrastructure can be damaged during fire incidents, causing system failure at critical moments.

- No Integration with Modern Technologies:

Existing systems lack integration with IoT, cloud platforms, and automation technologies, limiting their efficiency and functionality.

### PROPOSED SYSTEM

The proposed IoT system deploys distributed sensors (MQ-2 gas, DHT22 temperature/humidity, flame) interfaced with NodeMCU for Wi-Fi connectivity to a central server. Upon detection, it sends SMS/email alerts via GSM and authenticates users with fingerprint/RFID before siren activation or door unlocks. Cloud dashboards provide live feeds and historical data, with machine learning for predictive alerts based on patterns. This ensures 24/7 monitoring, minimizes false positives through dual-sensor fusion, and integrates with SCADA for automated extinguishers, offering 99% uptime and compliance with industrial safety norms.

Figure 2.4.1: Block Diagram of IOT Based Fire Alarming and Authentication System for Workhouse

#### Advantages of the Proposed System:

The proposed IoT Based Fire Alarming and Authentication System for Workhouse offers several advantages over traditional systems by integrating advanced technologies for improved safety, security, and efficiency:

- Early Fire Detection:

The use of multiple sensors such as smoke, temperature, and gas sensors enables early detection of fire hazards, reducing the risk of severe damage.

- Real-Time Monitoring:

The system allows continuous monitoring of environmental conditions through IoT, enabling users to access data remotely from anywhere.

- Instant Alert Notifications:

Alerts are sent immediately to authorized personnel via SMS, email, or mobile applications, ensuring a quick response during emergencies.

- Enhanced Security through Authentication:

The integration of RFID or biometric authentication restricts access to authorized users only, preventing unauthorized entry and system tampering.

- Reduced Response Time:

Automated alerts and responses significantly decrease the time taken to react to fire incidents, thereby minimizing potential losses.

- Automated Emergency Response:

The system can automatically activate alarms, buzzers, and fire suppression mechanisms such as sprinklers without manual intervention.

- Data Logging and Analysis:

Sensor data and event logs are stored in the cloud, allowing for analysis and improvement of safety measures over time.

- Scalability and Flexibility:

The system can be easily expanded by adding more sensors and devices, making it suitable for both small and large industrial environments.

- Cost-Effective Solution:

The use of low-cost IoT components such as NodeMCU reduces the overall system cost while maintaining high performance.

- High Reliability and Efficiency:

Continuous monitoring, secure authentication, and IoT integration improve the overall reliability and efficiency of the system.

### OBJECTIVES

The primary objective of the IoT Based Fire Alarming and Authentication System for Workhouse is to enhance workplace safety by providing early fire detection, secure authentication, and real-time alerting. The system aims to continuously monitor environmental parameters such as smoke, temperature, and gas levels using IoT-enabled sensors and instantly notify authorized personnel through cloud-based alerts. Another key objective is to restrict access to sensitive areas using authentication methods such as RFID or biometric verification, thereby preventing unauthorized entry and reducing safety threats. Overall, the system seeks to minimize fire-related risks, reduce response time during emergencies, and improve the overall safety management of workhouse environments.

### HARDWARE & SOFTWARE REQUIREMENTS

### HARDWARE REQUIREMENTS:

The IoT Based Fire Alarming and Authentication System for Workhouse requires the following hardware components for its implementation:

- NodeMCU (ESP8266/ESP32):

A microcontroller with built-in Wi-Fi capability used to process sensor data and enable IoT communication with cloud platforms.

- MQ-2 Gas Sensor:

Detects the presence of combustible gases and smoke in the environment, providing early indication of fire hazards.

- DHT22 Temperature and Humidity Sensor:

Measures temperature and humidity levels to identify abnormal environmental conditions that may lead to fire.

- Flame Sensor:

Detects the presence of fire or flame by sensing infrared light emitted by flames.

- RFID Module (RC522) / Fingerprint Sensor:

Used for authentication to allow access only to authorized personnel and enhance system security.

- LCD Display (16x2):

Displays system status, sensor readings, and alert messages for user interaction.

- Buzzer/Alarm:

Provides an audible alert when fire or abnormal conditions are detected.

- Relay Module:

Controls external devices such as sprinklers, exhaust fans, or alarm systems automatically.

- Power Supply Unit:

Provides a regulated DC power supply required for the operation of all components.

- Connecting Wires and Breadboard/PCB:

Used for circuit connections and prototyping of the system.

### SOFTWARE REQUIREMENTS:

The development and implementation of the IoT Based Fire Alarming and Authentication System for Workhouse require the following software components:

- Arduino IDE:

Used for writing, compiling, and uploading the program code to the NodeMCU (ESP8266/ESP32) microcontroller. It provides an easy-to-use environment for embedded system development.

- Embedded C / Arduino Programming Language:

The system is programmed using Embedded C through the Arduino IDE to control sensors, process data, and manage communication.

- Blynk / Firebase (Cloud Platform):

Used for real-time data monitoring, storage, and sending alerts to users through mobile or web applications.

- ESP8266/ESP32 Board Package:

Required in the Arduino IDE to support programming and interfacing with NodeMCU boards.

- Serial Monitor (Arduino IDE):

Used for debugging and testing by displaying real-time data and system responses.

- Mobile Application (Blynk App or Custom App):

Provides a user interface for monitoring sensor data, receiving alerts, and controlling the system remotely.

- MQTT Protocol (Optional):

Used for efficient data communication between devices and cloud servers in IoT-based systems.

# 3.SYSTEM ARCHITECTURE & DESIGN

## SYSTEM ARCHITECTURE & DESIGN

The IoT Based Fire Alarming and Authentication System for Workhouse is designed using a modular architecture that integrates sensing, processing, communication, and control units to ensure efficient fire detection and secure access control.

  - System Architecture

The system consists of multiple sensors such as gas (MQ-2), temperature and humidity (DHT22), and flame sensors, which continuously monitor environmental conditions. These sensors are connected to the NodeMCU microcontroller, which acts as the central processing unit.

The NodeMCU processes the data received from sensors and determines whether the conditions indicate a fire hazard. If abnormal values are detected, the system triggers alerts and activates safety mechanisms.

The processed data is transmitted to cloud platforms (such as Blynk or Firebase) through Wi-Fi, enabling real-time monitoring and remote access. Authorized users can view system status and receive notifications via mobile applications.

An authentication module, such as RFID or a fingerprint sensor, is integrated into the system to control access to restricted areas. Only authorized personnel can interact with the system or enter secured zones.

Additionally, output devices such as buzzers, alarms, relays, and LCD displays are connected to provide alerts, display information, and control external devices like sprinklers.

Figure 3.1: Project Architecture of IOT Based Fire Alarming and Authentication System for Workhouse

### DESCRIPTION

The system architecture diagram represents the overall working structure of the IoT-Based Fire Alarming and Authentication System in a simple block diagram format. The diagram illustrates the flow of data and interaction between various components of the system.

At the initial stage, multiple sensors such as the gas sensor, temperature sensor, and flame sensor are used to monitor environmental conditions continuously. These sensors detect parameters like smoke, heat, and fire presence and send the collected data to the central processing unit.

The NodeMCU (ESP8266/ESP32) acts as the core component of the system. It receives input data from the sensors, processes it, and determines whether a fire hazard exists. Based on the analysis, appropriate actions are taken.

An authentication module, such as an RFID or fingerprint sensor, is connected to the system to ensure that only authorized users can access or control the system. This enhances security and prevents unauthorized operations.

The processed data is then transmitted to a cloud server using Wi-Fi connectivity. Cloud platforms such as Blynk or Firebase are used for storing data and enabling real-time monitoring. Users can access system information remotely through mobile devices or computers.

In case of fire detection, the system activates output devices such as a buzzer or alarm to provide immediate alerts. Additionally, a relay module is used to control external devices like sprinklers or emergency systems.

An LCD display is included in the system to show real-time sensor readings and system status for local monitoring.

Overall, the diagram clearly shows how sensing, processing, communication, authentication, and alert mechanisms are integrated to form a complete and efficient fire safety system.

### DATA FLOW DIAGRAM

The Data Flow Diagram (DFD) represents the flow of data within the IoT Based Fire Alarming and Authentication System for Workhouse. It illustrates how data is collected, processed, stored, and transmitted between different components of the system.

Level 0 DFD (Context Diagram)

At the highest level, the system is represented as a single process interacting with external entities. The primary external entities include:

- User/Authorized Personnel: Receives alerts and monitors system status through mobile or web applications.

- Sensors (Gas, Temperature, Flame): Provide input data regarding environmental conditions.

- Authentication Module (RFID/Fingerprint): Sends user identity data for verification.

- Cloud Server: Stores data and facilitates remote access.

The system receives input data from sensors and authentication modules, processes it, and sends output in the form of alerts, notifications, and control signals.

Benefits:

The implementation of a Data Flow Diagram (DFD) in the IoT Based Fire Alarming and Authentication System for Workhouse offers several advantages:

- Clear Understanding of System Operation:

DFD provides a visual representation of how data moves through the system, making it easier to understand the overall functionality.

- Improved System Design:

It helps in identifying the relationship between different components, leading to better system planning and structured design.

- Efficient Data Management:

Ensures proper handling of input, processing, storage, and output data within the system.

- Error Identification:

Makes it easier to detect missing data, redundant processes, or incorrect data flow during system development.

- Simplifies Communication:

Helps developers, designers, and users to clearly understand the system without needing deep technical knowledge.

- Supports System Development:

Acts as a foundation for coding, testing, and implementation of the system.

- Enhances Documentation Quality:

Provides a structured and professional way to represent system processes in project documentation.

Applications:

The concept of data flow in this system can be applied in various real-world scenarios:

- Industrial Safety Systems:

Used in factories and warehouses for monitoring environmental conditions and ensuring fire safety.

- Smart Buildings:

Applied in offices and commercial buildings for automated fire detection and alert systems.

- Home Automation Systems:

Helps in developing smart home safety systems with remote monitoring and alerts.

- Security and Access Control Systems:

Used in systems that require authentication, such as RFID or biometric-based access.

- Healthcare Monitoring Systems:

Can be adapted for monitoring patient conditions and sending alerts in hospitals.

- Environmental Monitoring Systems:

Used for tracking air quality, temperature, and hazardous gas levels in different environments.

Levels of DFD:

The Level 1 DFD provides a detailed view of internal processes within the system. It includes the following main processes:

- Data Collection Process:

Sensors collect environmental data such as temperature, smoke, and gas levels and send it to the microcontroller.

- Data Processing Process:

The NodeMCU processes the incoming data to detect abnormal conditions indicating fire hazards.

- Authentication Process:

User data from RFID or biometric sensors is verified to allow or deny access.

- Alert Generation Process:

If fire conditions are detected, the system generates alerts and sends notifications to users.

- Data Storage Process:

All sensor data and event logs are stored in the cloud for monitoring and analysis.

- Control Process:

Based on system decisions, output devices such as alarms, buzzers, and relays are activated.

Figure 3.2: Dataflow Diagram  of  IOT Based Fire Alarming and Authentication System for Workhouse

# IMPLEMENTATION

4.IMPLEMENTATION

The implementation of the IoT Based Fire Alarming and Authentication System for Workhouse involves the integration of hardware components, embedded programming, and cloud-based communication to achieve real-time monitoring and secure access control.

  - System Setup

The system is built using a microcontroller (NodeMCU/ESP8266) which acts as the central processing unit. Various sensors such as smoke sensor (MQ-2), temperature and humidity sensor (DHT22), and flame sensor are connected to the microcontroller. These sensors continuously monitor environmental conditions inside the workhouse.

  - Sensor Integration

All sensors are interfaced with the NodeMCU through GPIO pins:

- MQ-2 sensor detects smoke and gas leakage

- DHT22 sensor measures temperature and humidity

- Flame sensor detects fire presence

The sensor data is read periodically and processed by the microcontroller.

  - Communication Module

NodeMCU has an inbuilt Wi-Fi module which is used to connect the system to the internet. The collected data is transmitted to cloud platforms such as Blynk or Firebase. This enables remote monitoring and real-time data access.

  - Alert Mechanism

When the sensor values exceed predefined threshold levels:

- A buzzer or siren is activated locally

- Notifications are sent to authorized users via mobile app, SMS, or email
This ensures immediate awareness and faster response during fire emergencies.

  - Authentication System

To enhance security, authentication modules like RFID or fingerprint sensors are integrated:

- Only authorized users can access restricted areas

- Unauthorized access attempts are denied and logged

- Authentication helps prevent system misuse and false alarms

  - Data Logging and Monitoring

All sensor readings and events are stored in the cloud database:

- Enables real-time monitoring through dashboards

- Helps in analyzing past incidents

- Improves decision-making and safety planning

  - Automation Features

The system can be extended to trigger automatic responses such as:

- Activating water sprinklers

- Unlocking emergency exits

- Turning off electrical supply

These actions reduce damage and enhance safety during emergencies.

- Software Implementation

The system is programmed using Arduino IDE:

- Embedded C/C++ is used for coding

- Libraries for sensors and Wi-Fi communication are included

- Logic is implemented for threshold detection and alert generation

4.7.1 Testing and Deployment

The system is tested under different conditions:

- Smoke and heat simulation

- Authentication verification

- Alert notification testing

After successful testing, the system is deployed in the workhouse environment for continuous monitoring.

### ALGORITHMS USED

### The IoT Based Fire Alarming and Authentication System uses a combination of simple decision-making and authentication algorithms to ensure efficient fire detection, alerting, and secure access control.

#### Fire Detection Algorithm

#### This algorithm is responsible for identifying fire hazards by continuously monitoring environmental parameters such as smoke, temperature, and flame intensity.

Algorithm Steps:

- Start the system

- Initialize all sensors (MQ-2, DHT22, Flame Sensor)

- Read sensor values at regular intervals

- Compare sensor values with predefined threshold levels

- If any sensor value exceeds the threshold:

      - Detect fire condition

      - Activate alarm (buzzer/siren)

      - Send alert notification to users

- Store the event data in the cloud database

- Repeat the process continuously

#### Multi-Sensor Decision Algorithm

To improve accuracy and minimize false alarms, the system uses a combination of multiple sensor inputs.

Algorithm Steps:

- Collect data from all sensors

- Evaluate conditions:

      - High temperature AND smoke detected

      - OR flame detected

- If conditions are satisfied:

Confirm fire event

- Else:

Ignore as non-critical condition

- Trigger alert only when fire is confirmed

#### Authentication Algorithm

This algorithm ensures secure access control by allowing only authorized personnel to interact with the system or access restricted areas.

Algorithm Steps:

- User provides input (RFID card / fingerprint)

- System reads the input data

- Compare input with stored database

- If match found:

      - Grant access

      - Allow system operation

- Else:

      - Deny access

      - Log unauthorized attempt

#### Alert Notification Algorithm

This algorithm manages communication between the system and users through IoT platforms.

Algorithm Steps:

- Detect fire condition

- Establish Wi-Fi connection

- Send sensor data to cloud server

- Generate alerts:

      - Mobile application notification

      - SMS or Email alert

- Update real-time dashboard

#### Automation Control Algorithm

This algorithm handles automatic emergency responses during fire incidents.

Algorithm Steps:

- Confirm fire detection

- Activate safety mechanisms:

      - Turn ON sprinkler system

      - Activate alarm

      - Unlock emergency exits

- Shut down electrical systems (if required)

- Continue monitoring until normal conditions are restored

To implement this project we have designed following modules:

- Sensor Module

The sensor module is responsible for detecting environmental changes that may indicate a fire hazard. It includes sensors such as the MQ-2 gas sensor, DHT22 temperature and humidity sensor, and flame sensor. These sensors continuously monitor parameters like smoke, gas leakage, temperature rise, and fire presence, and send the data to the microcontroller for

processing.

- Processing Module

The processing module consists of the NodeMCU (ESP8266) microcontroller, which acts as the brain of the system. It receives input data from all sensors, processes the information, and makes decisions based on predefined conditions. It also controls the activation of alarms,

communication modules, and other output devices.

- Communication Module

This module is responsible for transmitting data from the system to the cloud and users. Using the built-in Wi-Fi of NodeMCU, the system sends real-time data to IoT platforms such as Blynk or Firebase. It ensures that users can monitor the system remotely and receive instant alerts during emergencies.

- Alerting Module

The alerting module provides immediate notification when a fire hazard is detected. It includes components like buzzers, sirens, and LEDs for local alerts. Additionally, it sends notifications to users through mobile applications, SMS, or email, ensuring quick response and action.

- Authentication Module

The authentication module ensures system security by restricting access to authorized users only. It uses technologies such as RFID cards or fingerprint sensors. When a user attempts access, the system verifies their identity and grants or denies permission accordingly.

- Display Module

The display module (LCD) is used to show real-time information such as temperature, gas levels, system status, and alert messages. It helps users monitor system conditions directly at the device location.

- Power Supply Module

This module provides the required power to all components in the system. It includes a regulated power supply circuit that converts AC to stable DC voltage, ensuring proper functioning of the microcontroller, sensors, and other hardware components.

- Automation Module

The automation module handles automatic safety actions during fire incidents. It can activate sprinklers, unlock emergency exits, or shut down electrical systems to minimize damage and enhance safety without human intervention

### 4.2	SOFTWARE SAMPLE CODE DESCRIPTION

### This program is developed using Arduino IDE for implementing the IoT-Based Fire Alarming and Authentication System. It controls sensors, alerts, and display modules using a microcontroller.

#include <Servo.h>

Servo myservo;

#include <LiquidCrystal.h>

#include <stdio.h>

#include <SoftwareSerial.h>

SoftwareSerial mySerial(A4, A5);

LiquidCrystal lcd(6, 7, 5, 4, 3, 2);

// defines pins numbers

int fire      = 9;

int red_led   = 11;

int green_led = 10;

int buzzer    = 13;

int rtr1=0;

int dist1=0,dist2=0,dist3,sts1=0,sts2=0;

long duration;

int distanceCm, distanceInch;

unsigned char rcv,count,gchr='x',gchr1='x',robos='s';

char pastnumber[10];

char gpsval[50];

// char dataread[100] = "";

// char lt[15],ln[15];

int i=0,k=0;

int  gps_status=0;

float latitude=0;

float logitude=0;

String Speed="";

String gpsString="";

char *test="$GPRMC";

int hbtc=0,hbtc1=0,rtrl=0;

unsigned char gv=0,msg1[10],msg2[11];

float lati=0,longi=0;

unsigned int lati1=0,longi1=0;

unsigned char flat[5],flong[5];

unsigned char finallat[8],finallong[9];

int ii=0;

float tempc=0,weight=0;

float vout=0;

String inputString = "";         // a string to hold incoming data

boolean stringComplete = false;  // whether the string is complete

void okcheck()

{

unsigned char rcr;

do{

rcr = Serial.read();

}while(rcr != 'K');

}

void setup()

{

Serial.begin(9600);//serialEvent();

pinMode(fire, INPUT);

pinMode(red_led, OUTPUT);pinMode(green_led, OUTPUT);

pinMode(buzzer, OUTPUT);

myservo.attach(8);

myservo.write(10);

digitalWrite(red_led, HIGH);digitalWrite(green_led, HIGH);

digitalWrite(buzzer, HIGH);

delay(1500);

lcd.begin(16, 2);lcd.cursor();

lcd.print("IOT Fire Alarm");

lcd.setCursor(0,1);

lcd.print("Authentication");

delay(2000);

lcd.clear();

lcd.print("Fire:");//6,0

}

int dista=0;

int angle=0;

void loop()

{

if(digitalRead(fire) == LOW)

{

lcd.setCursor(6,0);lcd.print("ON  ");

digitalWrite(red_led, LOW);digitalWrite(green_led, HIGH);

digitalWrite(buzzer, LOW);

myservo.write(110);delay(5000);

while(digitalRead(fire) == LOW);

}

if(digitalRead(fire) == HIGH)

{

lcd.setCursor(6,0);lcd.print("OFF ");

digitalWrite(red_led, HIGH);digitalWrite(green_led, LOW);

digitalWrite(buzzer, HIGH);

myservo.write(10);delay(1500);

while(digitalRead(fire) == HIGH);

}

delay(1000);

}

void serialEvent()

{

while (Serial.available())

{

char inChar = (char)Serial.read();

if(inChar == '*')

{

// gchr = Serial.read();

gchr = 's';

}

}

}

void converts(unsigned int value)

{

unsigned int a,b,c,d,e,f,g,h;

a=value/10000;

b=value%10000;

c=b/1000;

d=b%1000;

e=d/100;

f=d%100;

g=f/10;

h=f%10;

a=a|0x30;

c=c|0x30;

e=e|0x30;

g=g|0x30;

h=h|0x30;

Serial.write(a);

Serial.write(c);

Serial.write(e);

Serial.write(g);

Serial.write(h);

}

void convertl(unsigned int value)

{

unsigned int a,b,c,d,e,f,g,h;

a=value/10000;

b=value%10000;

c=b/1000;

d=b%1000;

e=d/100;

f=d%100;

g=f/10;

h=f%10;

a=a|0x30;

c=c|0x30;

e=e|0x30;

g=g|0x30;

h=h|0x30;

//lcd.write(a);

lcd.write(c);

lcd.write(e);

lcd.write(g);

lcd.write(h);

}

void convertk(unsigned int value)

{

unsigned int a,b,c,d,e,f,g,h;

a=value/10000;

b=value%10000;

c=b/1000;

d=b%1000;

e=d/100;

f=d%100;

g=f/10;

h=f%10;

a=a|0x30;

c=c|0x30;

e=e|0x30;

g=g|0x30;

h=h|0x30;

// lcd.write(a);

// lcd.write(c);

// lcd.write(e);

// lcd.write(g);

lcd.write(h);

}

# 5.RESULTS & DISCUSSION

## RESULTS & DISCUSSION

The IoT Based Fire Alarming and Authentication System was successfully implemented using various hardware components such as sensors, NodeMCU, LCD display, buzzer, LEDs, and a servo motor. The system was tested under real-time conditions to evaluate its performance in detecting fire hazards and providing alerts. During testing, the system was able to detect fire conditions accurately. When the fire sensor was triggered, the buzzer was activated immediately, the red LED turned ON indicating danger, and the LCD displayed “Fire ON” to inform the user. At the same time, the servo motor rotated to a predefined angle, simulating an automatic response such as opening an emergency exit.

Under normal conditions, the system operated smoothly with the green LED turned ON, the buzzer turned OFF, and the LCD displaying “Fire OFF.” The servo motor remained in its default position. The system showed quick response time and continuous operation without any interruption, demonstrating its reliability in real-time monitoring.

The overall performance of the system was efficient and user-friendly, as it provided clear visual, audio, and display indications during fire conditions. The use of simple and cost-effective hardware components makes the system suitable for practical implementation in workhouse environments. However, the system relies on basic sensor input, which may be affected by environmental factors, leading to occasional inaccuracies. This limitation can be improved by integrating additional sensors and advanced technologies.

In conclusion, the system proved to be reliable, responsive, and effective for real-time fire detection and alerting, thereby enhancing safety and reducing risks in industrial environments.

# 6.VALIDATION

## VALIDATION

The IoT Based Fire Alarming and Authentication System was validated through a series of practical tests to ensure its accuracy, reliability, and proper functioning. The system was tested under different environmental conditions by simulating fire scenarios using heat and smoke sources. During these tests, the fire sensor successfully detected abnormal conditions and triggered the alert mechanisms without delay.

The buzzer and LED indicators responded correctly, and the LCD display showed the appropriate status messages such as “Fire ON” and “Fire OFF.” The servo motor also functioned as expected by rotating during fire detection, indicating an automated response. The system was further tested for normal conditions, where it maintained stable operation without generating false alarms.

Repeated testing confirmed that the system provides consistent performance and quick response time. The validation process also ensured that all hardware components are properly integrated and functioning together efficiently. Although minor variations may occur due to environmental factors, the overall system performance was found to be reliable and suitable for real-time fire safety applications.

### INTRODUCTION

Validation is the process of verifying whether the developed system meets the required objectives and performs accurately under different conditions. In this project, the IoT Based Fire Alarming and Authentication System was validated by conducting real-time tests to check its performance, reliability, and response time.

The system was tested by simulating fire conditions and observing the behavior of sensors, alert mechanisms, and output devices. The purpose of validation is to ensure that the system can detect fire hazards effectively, provide timely alerts, and operate continuously without failure. It also helps in identifying any limitations or errors in the system and improving overall efficiency.

6.2	TEST CASES

TABLE 6.2.1    FIRE DETECTION

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | Fire Detection | To detect fire using sensor | Fire sensor exposed to flame/heat | Buzzer ON, Red LED ON, LCD shows “Fire ON” |

TABLE 6. 2.2    NORMAL CONDITION

| Test case ID | Test case name | Purpose | Input | Output |
| --- | --- | --- | --- | --- |
| 1 | Normal Condition | To check system in safe condition | No fire or heat present | Green LED ON, LCD shows “Fire OFF” |
| 2 | Classification test 2 | To check if the classifier performs its task | Inappropriate- Content file is selected. | Inappropriate Content |

TABLE 6.2.3 RESPONSE TIME

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | Response Time | To check system response speed | Sudden fire exposure | Immediate alert within few seconds |

TABLE 6.2.4 CONTINUOUS MONITORING

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | Continuous Monitoring | To verify system stability | System runs continuously | System works without interruption |

TABLE 6.2.5 FALSE ALARM CHECK

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | False Alarm Check | To avoid false triggering | Minor environmental changes | No false alarm generated |

# 7.CONCLUSION & FUTURE ASPECTS

## 7. CONCLUSION & FUTURE ASPECTS

### 7.1 PROJECT CONCLUSION

The IoT Based Fire Alarming and Authentication System for Workhouse provides a reliable and intelligent solution to address the growing safety challenges in industrial and workplace environments. By integrating IoT technology with fire detection sensors and authentication mechanisms, the system ensures early identification of fire hazards and secure access control. Real-time monitoring and instant alerting significantly reduce response time, thereby minimizing potential damage to life and property.

Unlike traditional systems, the proposed solution offers remote accessibility, automated alerts, and centralized data management, making it more efficient and scalable. The inclusion of authentication enhances security by preventing unauthorized access and reducing the chances of system misuse or false alarms. Additionally, cloud-based data logging supports analysis and future improvements in safety management.

Overall, this system represents a modern, cost-effective, and scalable approach to workplace safety and can be effectively implemented in industrial environments.

### 7.2 FUTURE ASPECTS

In the future, the system can be further enhanced by integrating advanced technologies such as machine learning for intelligent fire prediction and improved decision-making. Additional sensors like gas, temperature, and humidity sensors can be incorporated to increase accuracy and reduce false alarms.

The system can also be integrated with automatic fire suppression systems such as water sprinklers for faster emergency response. Connecting the system directly to emergency services like fire stations can further improve safety measures. Development of a mobile application with real-time monitoring dashboards and notifications can enhance user interaction and control.

Furthermore, the use of long-range communication technologies and renewable energy sources such as solar power can make the system more efficient, scalable, and suitable for large industrial environments.

# 8.BIBLIOGRAPHY

## BIBLIOGRAPHY

### REFERENCES

- Duraivel, A. V., Wellington, B., Arul Nayagam, A., & Kijral, R. C. “An IoT Based Fire Alarming and Authentication System for Workhouse Using Raspberry Pi 3,” International Journal of Emerging Technology in Computer Science & Electronics, Vol. 25, Issue 5, April 2018.

- Guggilla, A., & Seshagiri Rao, V. “IoT Based Ware House Fire Safety System Using ARM7,” International Journal of Engineering and Technology, 2018.

- Al Hasani, I. M., Kazmi, S. I. A., Shah, R. A., & Hussain, S. “IoT Based Fire Alerting Smart System,” Sir Syed University Research Journal of Engineering & Technology, DOI:10.33317/ssurj.410.

- Gode, A. D., et al. “A Novel IoT Based Approach for Fire Warning System,” International Journal of Scientific Research in Science and Technology (IJSRST), Vol. 6, Issue 2, 2019.

- Shobha, T., & Sahoo, S. R. “Fire Detection Using IoT,” International Journal of Advanced Research in Computer and Communication Engineering (IJARCCE), 2024.

- Al Shereiqi, I. M., & Sohail, M. “Smart Fire Alarm System Using IoT,” Journal of Student Research, 2020.

- Gupta, P. K., & Singh, M. K. “A Secure Federated IoT-Based Multi-Sensor Framework for Intelligent Fire Detection and Alarm Systems,” International Journal of Innovative Science and Research Technology (IJISRT), 2025.

- “Internet of Things-based Fire Alarm Monitoring System for Industrial Electrical Panels,” Journal of Industrial Automation and Electrical Engineering, Vol. 2 No. 2, 2025.

- Ghadge, A. P., Sonawane, A., Belekar, V., & Gadade, M. “IoT Based Fire Detection and Control System,” International Journal of Scientific Research in Science, Engineering and Technology (IJSRSET), 2023.

- Tarhate, V., Prasad, G., Parate, A., & Shinde, A. “A Review on IoT Based Approach for Fire Monitoring System,” International Journal of Scientific Research in Computer Science, Engineering and Information Technology (IJSRCSEIT), 2020.

- Abdullahi, U. I., Zhang, W., Cao, Y., & Irankunda, G. “Integrating IoT Technology for Fire Risk Monitoring and Assessment in Residential Building Design,” Buildings, 2025, DOI:10.3390/buildings15081346.

- Jadon, A., Omama, M., Varshney, A., et al. “FireNet: A Specialized Lightweight Fire & Smoke Detection Model for Real-Time IoT Applications,” 2019.

- Hasib, A. et al. “Dual-Modality IoT Framework for Integrated Access Control and Environmental Safety Monitoring with Real-Time Cloud Analytics,” 2026.

- Imteaj, A., Rahman, T., Hossain, M. K., et al. “An IoT Based Fire Alarming & Authentication System for Workhouse Using Raspberry Pi3,” IEEE ECCE, 2017 (cited in related literature).

- Johnsaida, N., Rahul, L., & Shalini, T. “IoT Based Smart Fire Emergency Response System,” International Journal of Advanced Research and Development, 2018 (cited in review articles).

### GITHUB LINK

https://github.com/Manojkumar128/IOT-BASED-FIRE-ALARMING-AND-AUTHENTICATION-SYSTEM-FOR-WORKHOUSE.git